Luckfox Pico Ultra W

on 86 Panel Board

Hardware Configuration Guide

UART4_M0 + GT911 TouchScreen Fix

March 2026

# Overview

This guide documents the complete process of configuring a Luckfox Pico Ultra W module mounted on the Luckfox Pico 86 Panel board. Because no dedicated 86 Panel firmware exists, the standard Ultra W Ubuntu firmware must be patched manually.

Two subsystems are covered:

- UART4_M0 — enabling /dev/ttyS4 via config file (no GUI required)

- GT911 TouchScreen — fixing Device Tree (DTB) pin assignments and driver conflicts

| Item | Value |
| --- | --- |
| Board | Luckfox Pico 86 Panel |
| Module | Luckfox Pico Ultra W |
| OS | Ubuntu firmware (Ultra W) |
| Access | SSH only (no GUI/HDMI) |
| Config file | /etc/luckfox.cfg |
| Config tool | /usr/bin/luckfox-config |
| Touch IC | Goodix GT911 @ I2C3 (0x5d) |

# Part 1: Enabling UART4_M0

## Background

The luckfox-config tool opens a graphical TUI (ncurses) when invoked without arguments. To enable UART4 over SSH without a terminal emulator that supports full TUI, write the config file directly and call luckfox-config load (non-GUI mode).

⚠ Never use echo >> to append to /etc/luckfox.cfg — this causes binary/null byte corruption.

⚠ Always rewrite the entire file using cat > in a single operation.

## Known Conflicts

Do NOT include I2C3_M0_STATUS=1 when TS_ENABLE=1. These conflict and cause an error during load. The touchscreen overlay already handles I2C3 internally.

## Step-by-Step

- Write the config file (rewrite entire file):

sudo bash -c 'cat > /etc/luckfox.cfg << EOF

SPI0_M0_CS_ENABLE=1

SPI0_M0_MODE=1

TS_ENABLE=1

UART4_M0_STATUS=1

EOF'

- Apply the config (non-GUI, dynamic overlay — no reboot needed):

sudo luckfox-config load

- Verify UART4 appeared:

ls /dev/ttyS*

# Expected: /dev/ttyS4

✓ Auto-boot: /etc/rc.local already calls 'luckfox-config load' — no extra setup needed.

## UART4 Pin Mapping

| Signal | GPIO | Function |
| --- | --- | --- |
| RX | GPIO1_B0 | Receive |
| TX | GPIO1_B1 | Transmit |

# Part 2: GT911 TouchScreen Fix

## Problem Description

After enabling TouchScreen via luckfox-config and rebooting, /dev/input/event0 appears as Goodix Capacitive TouchScreen but does not respond to touch. The interrupt counter (gt911 in /proc/interrupts) stays at 0.

Root cause: The DTB shipped with Ultra W firmware has incorrect GPIO assignments for the 86 Panel's touch controller, and the audio codec occupies the same GPIO as the touch interrupt line.

## Hardware Pin Map (86 Panel Schematic)

| Signal | GPIO | DTB Property |
| --- | --- | --- |
| TP_INT (interrupt) | GPIO0_A0 | irq-gpios / interrupts |
| TP_RES (reset) | GPIO3_D0 (pin 24) | reset-gpios |
| TP_SDA | GPIO3_D1 (I2C3_M2) | — |
| TP_SCL | GPIO3_D2 (I2C3_M2) | — |

## Root Causes Found

### 1. Wrong interrupt pin

The original DTB had interrupts = <0x03 0x02> (GPIO0_A3). The correct pin is GPIO0_A0, so it must be interrupts = <0x00 0x02>.

### 2. Wrong reset GPIO

The original DTB had reset-gpios = <0x3b 0x04 0x01> which maps to GPIO0_A4. The correct pin is GPIO3_D0 (pin 24 of gpio3 bank). The correct value is reset-gpios = <PHANDLE_GPIO3 0x18 0x01>.

⚠ The phandle for gpio3 changes after luckfox_fdt_overlay runs. Always read it fresh from the DTB in /dev/mmcblk0p4 — do not hardcode it.

### 3. Missing irq-gpios property

The irq-gpios property was entirely absent from the touchscreen node. It must be added: irq-gpios = <PHANDLE_GPIO0 0x00 0x00>.

### 4. Audio codec conflict on GPIO0_A0

The audio codec node (acodec @ 0xff480000) had pa-ctl-gpios = <PHANDLE_GPIO0 0x00 0x00> — claiming GPIO0_A0 for PA (power amplifier) control. This caused the Goodix driver to fail with error -16 (EBUSY) because the GPIO was already held by the audio driver.

✓ Solution: Remove pa-ctl-gpios from the audio node. Audio recording/playback still works; only the external PA control pin is disabled.

## Step-by-Step Fix

### Step 1: Verify GT911 is reachable via I2C

First unbind the driver so I2C is free, then read the product ID register:

echo '3-005d' | sudo tee /sys/bus/i2c/drivers/Goodix-TS/unbind

sudo i2ctransfer -y 3 w2@0x5d 0x81 0x40 r6

# Expected: 0x39 0x31 0x31 ... (ASCII '911')

### Step 2: Extract and decompile the current DTB

sudo dd if=/dev/mmcblk0p4 bs=1 skip=2048 count=41815 of=/tmp/current.dtb 2>/dev/null

dtc -I dtb -O dts /tmp/current.dtb -o /tmp/current.dts 2>/dev/null

⚠ The count value (41815) must match the actual DTB size. Read it from the FDT header's data-size field if unsure.

### Step 3: Find correct phandles

Phandles are reassigned each time overlays are applied. Always look them up from the current DTB:

# Find gpio0 phandle

grep -A10 'gpio@ff380000' /tmp/current.dts | grep phandle

# Find gpio3 phandle

grep -A10 'gpio@ff550000' /tmp/current.dts | grep phandle

### Step 4: Patch the touchscreen node

Replace the incorrect values and add the missing irq-gpios property. In this session the phandles were: gpio0 = 0x3b, gpio3 = 0x3f.

# Fix reset-gpios (use your gpio3 phandle)

sed -i 's/reset-gpios = <OLD_VALUE>/reset-gpios = <0x3f 0x18 0x01>/' /tmp/current.dts

# Fix interrupt pin (GPIO0_A0 = pin 0)

sed -i 's/interrupts = <0x03 0x02>/interrupts = <0x00 0x02>/' /tmp/current.dts

# Add missing irq-gpios before pinctrl-names line

sed -i '/compatible = "goodix,gt911"/,/pinctrl-names/ {

/pinctrl-names/ i\                        irq-gpios = <0x3b 0x00 0x00>;

}' /tmp/current.dts

### Step 5: Remove audio PA conflict

sed -i '/pa-ctl-gpios = <0x3b 0x00 0x00>;/d' /tmp/current.dts

# Verify it is gone

grep 'pa-ctl-gpios' /tmp/current.dts  # must return nothing

### Step 6: Verify final touchscreen node

The node should look exactly like this (with your actual phandles):

touchscreen {

status = "okay";

compatible = "goodix,gt911";

reg = <0x5d>;

interrupt-parent = <0x3b>;   /* gpio0 */

interrupts = <0x00 0x02>;    /* GPIO0_A0, edge */

reset-gpios = <0x3f 0x18 0x01>; /* gpio3, pin 24, active-low */

irq-gpios = <0x3b 0x00 0x00>;   /* gpio0, pin 0 */

pinctrl-names = "default";

};

### Step 7: Compile and write patched DTB

dtc -I dts -O dtb /tmp/current.dts -o /tmp/patched.dtb 2>/dev/null

SIZE=$(stat -c%s /tmp/patched.dtb)

SIZE_HEX=$(printf '%x' $SIZE)

echo "Size: $SIZE (0x$SIZE_HEX)"

sudo dd if=/tmp/patched.dtb of=/dev/mmcblk0p4 bs=1 seek=2048 2>/dev/null

### Step 8: Update FDT header

The FIT image header stores the DTB size and SHA256 hash. Both must be updated:

SHA=$(sha256sum /tmp/patched.dtb | awk '{print $1}')

HASH_FORMATTED=$(echo $SHA | sed 's/.\{8\}/0x& /g')

cat > /tmp/fdt_hdr.dts << EOF

/dts-v1/;

/plugin/;

&{/images/fdt}{

data-size = <0x0000${SIZE_HEX}>;

hash{ value = <${HASH_FORMATTED}>; };

};

EOF

dtc -I dts -O dtb /tmp/fdt_hdr.dts -o /tmp/fdt_hdr.dtbo 2>/dev/null

sudo dd if=/dev/mmcblk0p4 bs=1 skip=0 count=2048 of=/tmp/fdt_header.dtb 2>/dev/null

fdtoverlay -i /tmp/fdt_header.dtb -o /tmp/fdt_header_new.dtb /tmp/fdt_hdr.dtbo

sudo dd if=/tmp/fdt_header_new.dtb of=/dev/mmcblk0p4 bs=1 seek=0 count=2048 2>/dev/null

echo 'Done'

### Step 9: Reboot and verify

sudo reboot

# After reboot:

dmesg | grep -i 'gt9\|goodix'

# Expected: Goodix-TS 3-005d: ID 911, version: 1060

cat /proc/interrupts | grep gt911

# Expected: counter increasing when touching

sudo evtest /dev/input/event0

# Touch the screen — events should appear

# Final Working State

## /etc/luckfox.cfg

SPI0_M0_CS_ENABLE=1

SPI0_M0_MODE=1

TS_ENABLE=1

UART4_M0_STATUS=1

## Confirmed Results

| Check | Command | Expected Result |
| --- | --- | --- |
| UART4 present | ls /dev/ttyS* | /dev/ttyS4 appears |
| GT911 on I2C | sudo i2cdetect -y 3 | 0x5d detected |
| Driver loaded | dmesg ¦ grep goodix | ID 911, version: 1060 |
| Interrupts active | cat /proc/interrupts ¦ grep gt911 | Counter increments on touch |
| Touch events | sudo evtest /dev/input/event0 | EV_ABS X/Y events on touch |
| GPIO0_A0 free | sudo cat /sys/kernel/debug/gpio | gpio-0 not listed (no pa-ctl) |

## Common Pitfalls

| Pitfall | Symptom | Fix |
| --- | --- | --- |
| echo >> to luckfox.cfg | Config binary corruption | Always use cat > to rewrite |
| I2C3 + TS_ENABLE conflict | Error during luckfox-config load | Remove I2C3_M0_STATUS when TS_ENABLE=1 |
| Hardcoded phandle | Wrong GPIO, probe fails -22 | Always read phandle from current DTB |
| pa-ctl-gpios conflict | Goodix probe fails error -16 | Remove pa-ctl-gpios from acodec node |
| DTB header not updated | Boot may use wrong size | Always run fdtoverlay header update |
| Dynamic overlay -22 | overlay phandle fixup failed | Use static DTB patch instead |

— End of Guide —